Comprehensive User Manual: Bridge Management System (BMS) for

GujRAMS

1. System Overview and Strategic Purpose

The Bridge Management System (BMS) serves as a critical pillar within the Gujarat Road Asset

Management System (GujRAMS), providing a centralized repository for the state’s extensive

infrastructure network. By digitizing physical attributes—including structure length, overall width

(m), and surface types—alongside detailed condition data such as cracking, bulging, and

scouring, the BMS enables stakeholders to transition from reactive maintenance to data-driven

strategic planning. Centralizing this data ensures a single "source of truth" across the GujRAMS

ecosystem, facilitating the long-term preservation of over 5,635 bridges (major and minor) and

4,050 culverts currently managed across the state’s State Highways (SH), Major District Roads

(MDR), and Village Roads (VR).The primary objectives of the BMS module are:

● Asset Documentation: Recording precise geographical coordinates and defining the

primary structural features of all bridge and culvert assets.

● Standardization: Establishing uniform engineering parameters for capturing inventory

and condition data to ensure statewide consistency.

● Lifecycle Management: Facilitating the full lifecycle of bridge data, from the initial

recording of new assets to the retirement of replaced or decommissioned structures.

● Structural Assessment: Calculating the Bridge Condition Index (BCI) as a quantitative

measure of structural health and safety.

● Strategic Prioritization: Generating rehabilitation measures and standardized reports

to inform multi-year budgeting and maintenance schedules.With these strategic

objectives established, the following sections detail the technical protocols for secure

system access and operation.

2. System Access and Security Protocols

Maintaining the integrity of state infrastructure data requires rigorous security measures.

GujRAMS utilizes a role-based access control framework, ensuring that only authorized

engineering personnel and administrators can modify sensitive asset records.

2.1 Authentication Workflow

Access to the BMS module is managed through the GujRAMS web application. To authenticate:

1. Navigate to the official URL: https://gujrams.gujarat.gov.in.

2. Enter your unique Login ID and Password (as provided by the system administrator).

3. Enter the alphanumeric CAPTCHA to validate the session.

4. Click Login to access the system Home Page.

2.2 Credential Recovery

In the event of forgotten credentials, users must follow the secure recovery protocol:

1. Select the Forgot Password link on the primary login screen.

2. Enter the registered Username .

3. Click the Reset button.

4. A secure reset link will be dispatched to the user’s registered official email address.

2.3 User Navigation and Interface Logic

The GujRAMS interface employs standardized iconography to ensure high usability for field

engineers and office-based consultants.| Icon | Name | Function || ------ | ------ | ------ || Navigator

| Module Navigator | Provides access to all active GujRAMS modules (BMS, PMS, TIS). ||

Theme | Theme Toggle | Adjusts the visual theme (Light/Dark) for user comfort. || Map | Map

Navigator | Redirects the user to the spatial GIS overview. || Template | Data Templates |

Enables download of Excel-based templates for bulk data imports. || Actions | Action Tools |

Specific icons for Add (+), Edit (pencil), and View (eye). || Data | Data Management | Functions

for Export (download) and the Gallery (images). |

The interface provides two primary operational perspectives:

● Map View: The default GIS-based display offering a spatial overview of assets. It

includes high-level Key Performance Indicators (KPIs) showing total bridge and culvert

counts across jurisdictions.

● Dashboard View: Accessible via the "Chart" toggle, this view presents KPI-based

analytics, including the distribution of bridge conditions and jurisdiction-specific asset

counts via pie and bar charts.This navigational framework provides the necessary

foundation for managing the system's operational core: Survey Data.

3. Survey Data Management

Granular survey data is the prerequisite for accurate condition modeling and maintenance

prioritization. Precise inventory and condition details allow the system to generate the

high-fidelity indices required for long-term investment planning.

3.1 Searching and Filtering Framework

The BMS manages a massive statewide dataset using a robust filtering engine. Users can

locate assets using Road Codes, Bridge Types, or specific Bridge Codes.

● Jurisdiction Filtering: Users can limit data grids to a specific "Circle" or "Division."

Selecting a jurisdiction automatically populates the subsequent dropdown menus with

only the relevant Road Codes.

● Logic Rules: The system supports "Match All" (AND logic) and "Match Any" (OR logic)

parameters. Users should click "Add Rule" to layer multiple filters.

● Execution: After defining criteria in the column-level filter icons (e.g., "Contains" or

"Starts With"), users must click the "Apply" button to refresh the grid. Clicking

"Clear" resets the grid to show the full dataset.

3.2 Bridge Inventory and Protection Works

Adding or modifying a bridge record requires comprehensive data entry across four mandatory

tabs to establish a complete structural profile:

1. Location Details: Captures Road Code, precise Latitude/Longitude, and R&BD

Chainage (km).

2. Span Details: Defines structural support systems (Simply Supported, Continuous, or

Cantilever) and specific span dimensions.

3. Details of Protection Works: Records attributes for embankments and batter

protection.

4. Top Deck: Details regarding the wearing coat and surface attributes.Upon saving the

location and chainage, the system automatically generates a Bridge Code . This unique

identifier serves as the primary key for all related data tables across the GujRAMS

ecosystem.

3.3 Specialized Substructures: Abutments and Piers

Substructure components are tracked with high precision to monitor foundation health:

● Abutments: Requires inventory and condition data for foundations, substructures, and

pipe structures.

● Piers: Tracks pier shafts, caps, and bearings.

● Efficiency Logic: To streamline the entry of identical components, users can use the

Copy/Paste function (represented by a paper icon) to replicate data across tabs.

3.4 Culvert and Committed Project Management

Culverts: Essential for drainage and road integrity, culverts are managed via a dedicated

section that captures structure types and link codes. Users may "Retire" culvert records if they

are replaced or permanently removed.Committed Projects: This category identifies

infrastructure works that are already approved or in progress.

● Strategic Importance: These projects must be accurately flagged and excluded during

budgeting and prioritization analysis. This "So What?" factor is critical; it prevents

"double-counting" in the Highway Development and Management (HDM) analysis,

ensuring maintenance funds are not mistakenly allocated to assets already scheduled

for intervention.This raw inventory and condition data forms the basis for the system’s

primary analytical output: the Bridge Condition Index.

4. Bridge Condition Index (BCI) Methodology

The Bridge Condition Index (BCI) is the strategic metric used to assess the structural and

functional health of an asset. It converts technical inspection data into a prioritized roadmap for

maintenance, repair, and replacement.

4.1 Condition Rating Logic and Interpretation

The system calculates BCI on a scale of 0 to 100. This numerical value dictates the required

engineering and financial response:| BCI Value Range | Maintenance Action / Status || ------ |

------ || 80 to 100 | Retained with Routine Maintenance || 65 to 80 | Minor Repairs Required || 45

to 65 | Moderate Repairs Required || 30 to 45 | Requires Strengthening / Rehabilitation || < 30 |

Re-construction / New Construction |

The system uses specific Condition Codes (e.g., A1: Excellent, B2: Fair, NE: Not Evaluated)

to rate sub-components. Furthermore, an Age Factor is applied to account for the natural

degradation of materials over the bridge's lifecycle.

4.2 Element-Wise Assessment (BECI)

The Final BCI is a weighted average of several BECIs (Bridge Element Condition Indices) .

Each structural element is assigned an "Importance Factor" reflecting its criticality:

● Super-Structure: Covers RC Solid Slabs or Girders (Importance Factor: 4.00 to 5.00).

● Sub-Structure: Abutment walls, Pier shafts, and caps (Importance Factor: 4.00).

● Foundations: Assessments of Scour and Settlement (Importance Factor: 5.00).

● Protection Works: Batter protection and retaining walls (Importance Factor:

2.00).Critical Engineering Thresholds:

● Widening Required: Triggered if Overall Width (OW) is < 8.0m for Minor Bridges or <

7.0m for Major Bridges.

● Super-structure Replacement (Rep. SS): Automatically recommended if the

Super-structure BECI falls below 30% , regardless of the overall BCI.These analytical

indices provide the data necessary for the advanced operations required for state-level

reporting.

5. Advanced Data Operations: Import, Export, and Reporting

To ensure data mobility and administrative control, the BMS provides specialized tools for bulk

management and standardized reporting.

5.1 Data Import and Template Management

For large-scale updates, users should utilize standardized Excel templates. The workflow

includes:

1. Downloading: Accessing the correct template (e.g., Approach Protection Details) via

the Data Templates icon.

2. Field Mapping: Aligning Excel columns with the system’s internal database schema

during the upload process.

3. Auditing: Reviewing the Error Log post-upload to identify and rectify data quality

issues before they are committed to the live database.

5.2 Exporting and the Visual Audit Trail

The system offers two methods for data extraction:

● Export Summary: A comprehensive export containing all data from the four primary

inventory tabs (Location, Span, Protection, Top Deck).

● Export All Data: A streamlined export of the current grid view, capturing only the visible

columns.

● The Gallery Function (Visual Audit Trail): Beyond simple image storage, the Gallery

serves as a Visual Audit Trail . It allows users to upload time-stamped images, NDT

test results, and inspection videos. These documents provide a legal and engineering

history of the asset’s condition over time, supporting long-term forensic engineering and

accountability.

5.3 Reporting and Lookup Data

Standardized reports, such as BI01 - Bridge Details , provide executive summaries for

management review. Administrators manage the system's global intelligence through the

Lookup Data section, which contains:

● Condition Severity Definitions: Parameters for the BCI engine.

● BOQ Master Lists: Standardized Bill of Quantities for repair works.

● Unit Rates: Current market rates for repair materials and labor.By diligently maintaining

these records and utilizing the BMS tools, users contribute directly to the continuous

improvement and long-term sustainability of Gujarat’s vital road assets